| Purpose: |  |  |  |  | To ensure the safe operation and use of the (Add Make/Model) Walkie Stacker at IGA XXXX |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Responsibilities: |  |  |  |  | Licensed and authority to operate |  |  |  |  |  |  |  |  |  |  |  |
| Personal Protective Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety Glasses | Gloves |  |  Safety Footwear |  |  | Ear plugs/Muffs | Face shields |  |  High Visibility Vest/Clothing | Safety Hat | Safety Harness | Protective Clothing |  | Safety Mask |  OTHER Cold environment protection as required | NO PPE REQUIRED |
| Pre-Start Checks |  |  |  |  |  |  |  |  |  |  |  |  | Potential Hazards |  |  |  |
| - High Risk Work License (Code LF) * - Completed IGA Competency process * - Dressed appropriately for work with correct PPE and no loose clothing - Australian standard safety boots - Fit for work - First aid kit on hand - Walkie Stacker prestart completed - Attachment points in good condition - Battery Charged - Work area ready, no spills and access clear to and from charging area. * Mandatory requirements. DO NOT operate the stacker without these controls in place. |  |  |  |  |  |  |  | ADD PICTURE OF WALKIE STACKIE |  |  |  |  | - Manual handling/repetitive strains - Unstable/overloaded loads - Incorrect staking - Objects fall off tynes - Tip over due to sharp turning - Tip forward due to travel with mast high - Failure of plant resulting in loss of content, loss of load - Damaged pallets/bins - Poor visibility/ obstruction/ blind spots - Travelling turning on gradients - Stacker impact on structure - Rain/spills/ causing slippery conditions - Poor work methods/practices - Battery explosion/Chemical burn - Other vehicle traffic - Untrained in safe work procedures - Pedestrians/bystanders - No license |  |  |  |
| Procedure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Steps |  | Instructions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 Plan the work |  |  |  | - Ensure the pre-start checks listed above have been completed. - Ensure the work area is in a suitable condition for the proposed tasks (see hazard list) - Locate and ensure you are familiar with all machine operations and controls. |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 Operational condition checks |  |  |  | - Ensure the stacker pre-start inspection is completed and signed off by the Store Manager. - Undertake your own checks before operation: - Ensuring the stacker is on flat ground - Inspecting the stacker’s body for damage and hydraulic for breakages and leaks - Checking safety devices. - Verifying that you have 360-degree visibility before operation - Removing loose items from the stacker - Finally, check the horn, steering, and brakes before operation. - Record and report any faults to your Store Manager. - Tag out for prohibited use if any safety features are non-functional. |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 Transporting Loads |  |  |  | - View the area and surrounding before moving off, ensure it is safe to commence work. - While moving around the work area pay attention to team members on foot and ensure you know their positions (2 meters away from stacker). - Ensure you follow the guidelines of signage for each work area. - Ensure you reduce speed and sound horn when passing through doors and blind corners. - Maintain speed limits depending on the environment and reduce speed when in doubt. Especially travelling through doors - Carry loads at the lowest possible height to maximise stability. This must be 100mm or 10cm. - Loads must not be more than a 3-bin or 15 pallets stack - Ensure you use reverse travel when the load is tall enough to obscure forward vision. - Reduce speed and acknowledge personnel via horn or communication when operating in the same area - Ensure mast is slightly tilted back when carrying loads, this shifts centre of gravity to a more stable position. - The horn must be sounded with 2 toots when approaching corners or blind corners as well as any blind spot where vision of intended path is obstructed. - Do not lift loads over any person. - Cease operation if persons on foot approaches the machine. Apply park position. |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 Shut down and park |  |  |  | - Once work is completed, return the stacker to the designated parking area. - When parking - Use the handbrake - Lower forks to the ground - Tilt so tips touch - Remove key and return to office safe - If any faults have been identified while working, record and report them. - Tag out the machine for prohibited use if any faults are found with safety features. |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 Recharge |  |  |  | - Return the stacker to the recharging station. - Parked position tynes tilted forwards - Follow the charging SWP to correctly and safely recharge the machine. |  |  |  |  |  |  |  |  |  |  |  |  |
| Dos and Don’ts |  |  |  | - A stacker must not be used as a towing or push device, unless appropriate attachments are fitted - A tow rope must never be attached to the mast to pull or drag loads - The stacker must never travel with tynes raised or reach mechanism extended - A person should not push on the point of one or both tynes. Nor should a person stand or walk under the elevated tynes, even when a load is not being carried - A stacker must not lift loads greater than the capacity of the stacker - A stacker must not lift a load that appears unsuitable or unstable (including on a damaged pallet/or bin) - Don’t prematurely raise loads when approaching trucks, tailgates and racking - Don’t turn whilst raising loads - A stacker must not be parked on an incline or operated on gradients with the load elevated - Do check over both shoulders before and whilst reversing - Re-adjust speed when turning under load or otherwise must be safe and reasonable - Make sure you understand the compliance plate of the stacker rating - Stackers must only be operated on IGA premises. Not on public roads with an exception noted for placing pallets on nature strip - There must be a strictly enforced no Smoking Policy in battery charging area - Naked flames should not be used when checking the level of electrolyte in battery cells. |  |  |  |  |  |  |  |  |  |  |  |  |

| Training |  | trainee I acknowledge and understand the SWP and the training I have received. Trainer: The trainee has been told and shown what a compliant standard looks like when conducting this task safely, the key risks have been emphasised, and the requirements of the process have been clearly communicated. |  |  |
| --- | --- | --- | --- | --- |
| Trainee Name: |  | Trainee Signature: Payroll Number: Date: |  |  |
| Trainer Name: |  | Trainer Signature: Payroll Number: Date: |  |  |
| Competency | trainee: I have the knowledge and skills to undertake this task confidently and safely. Trainer: The trainee has demonstrated confidence, understanding and competency in conducting this task safely. |  |  |  |
| Trainee Name: |  |  | Trainee Signature: | Date: |
| Competent Person: |  |  | Competent Person Signature: | Date: |