| Purpose: |  |  |  |  |  |  | To ensure the safe use of the Deli slicer. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Responsibilities: |  |  |  |  |  |  | All team members who use the meat slicer. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Personal Protective Equipment – As Required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety Glasses |  |  Gloves |  |  Safety Footwear |  |  |  | Ear plugs/Muffs |  | Face shields |  |  High Visibility Vest/Clothing | Safety Hat | Safety Harness | Protective Clothing | Safety Mask |  |  | OTHER Cold and Weather protection as required | NO PPE REQUIRED |  |
| Pre Start Checks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Potential Hazards |  |  |  |  |
| - Dressed appropriately for work with correct PPE - Sensible closed shoes with rubber soles - Non-Latex Gloves - Work area clear to operate safely - Floor clean and free of slippages - Fit for work - First aid kit on hand |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - Sharp rotating blade - Electricity - Slips and Trips - Heavy objects - Cleaning/blockages |  |  |  |  |
| Procedure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Steps |  |  | Instructions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 Plan the work |  |  |  |  | - DO NOT operate the machinery without training - Ensure you are wearing any required PPE (gloves, apron, closed shoes). - Check the meat slicer, ensure it is clean and operational with no blockages. - Ensure any electrical leads are in good condition and not in a position to be cut or damaged. - Ensure you have enough room for the products and trays or containers ready to receive the slices. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 Conduct the work |  |  |  |  | - Use the meat slicer to slice the stock as required. - Always use the meat holder/pusher to move the meat, never use your hands. - Always keep your hands behind the guards and on the correct handles. - DO NOT catch the slices with your hands - Finish slicing the stock and remove the product and the waste from the slicer. - Turn off the meat slicer when changing foods |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 Finish up |  |  |  |  | - Switch off the slicer at the machine switch and the wall. Unplug if you intend to wash the slicer down. - Return the blade to zero to ensure the blade is aligned with the guard. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 Dismantle and clean |  |  |  |  | - Turn the power off before cleaning - Following manufacturer’s instructions, remove the parts of the slicer that require cleaning. - Clean and sanitize slicing machine. - With the Slicer, dismantle wash off all excess material and rinse with portable water . Wash with Microclean (1:50) - Never sharpen a dirty blade - Re-assemble the slicer. - If faults or blockages cannot be removed Lock and Tag out slicer and report issue. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 Check and finish |  |  |  |  | - Once the slicer is re-assembled, turn it on and check it is working and ready for the next job. - If faults or blockages cannot be removed to Lock and Tag out slicer and report issue. - Turn the slicer off. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Training |  |  |  |  |  |  |  | trainee I acknowledge and understand the SWP and the training I have received. Trainer: The trainee has been told and shown what a compliant standard looks like when conducting this task safely, the key risks have been emphasised, and the requirements of the process have been clearly communicated. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Trainee Name: |  |  |  |  |  |  |  | Trainee Signature: Payroll Number: Date: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Trainer Name: |  |  |  |  |  |  |  | Trainer Signature: Payroll Number: Date: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Competency |  |  |  |  | trainee: I have the knowledge and skills to undertake this task confidently and safely. Trainer: The trainee has demonstrated confidence, understanding and competency in conducting this task safely. Training topics has included: - Never put a hand behind the food chute guard. - Never place food into the slicer by hand-feeding or hand pressure. - Use a pushing/guarding device with chute-fed slicers. - Use plungers to feed food into chute-fed slicers or use the feeding attachment located on the food holder. - Turn off the food slicer when changing foods. - Keep hands out of the danger zone on the back of the blade where the sliced food exits. - Turn off and unplug slicers when not in use or when left unattended for any amount of time. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Trainee Name: |  |  |  |  |  |  |  |  |  |  | Trainee Signature: |  |  |  |  |  | Date: |  |  |  |
|  | Trainee Name: |  |  |  |  |  |  |  |  |  |  | Trainee Signature: |  |  |  |  |  | Date: |  |  |  |
|  | Trainee Name: |  |  |  |  |  |  |  |  |  |  | Trainee Signature: |  |  |  |  |  | Date: |  |  |  |
|  | Trainee Name: |  |  |  |  |  |  |  |  |  |  | Trainee Signature: |  |  |  |  |  | Date: |  |  |  |
|  | Trainee Name: |  |  |  |  |  |  |  |  |  |  | Trainee Signature: |  |  |  |  |  | Date: |  |  |  |
|  | Competent Person: |  |  |  |  |  |  |  |  |  |  | Competent Person Signature: |  |  |  |  |  | Date: |  |  |  |