| Purpose: |  |  |  |  | To ensure the safe use of open fryer |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Responsibilities: |  |  |  |  | All personnel authorized to use the open fryer |  |  |  |  |  |  |  |  |  |  |  |
| Personal Protective Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  Safety Glasses |  Gloves |  |  Safety Footwear |  |  | Ear plugs/Muffs | Face shields |  | High Visibility Vest/Clothing | Safety Hat | Safety Harness | Protective Clothing | Safety Mask |  | OTHER Cold and Weather protection as required | NO PPE REQUIRED |
| Pre Start Checks |  |  |  |  |  |  |  |  |  |  |  |  |  | Potential Hazards |  |  |
| - Dress appropriately for work with correct PPE - Non slip closed shoes, no loose clothing - Tie hair back and wear a hat - Heat proof gloves* - Safety glasses * - Fit for work - First aid kit on hand *For use, dispensing and emptying Open Fryer |  |  |  |  |  |  |  |  |  |  |  |  |  | - Fire - burns from hot oil - contact with hot surfaces - Fumes from boiling cleaning chemicals - Boiling chemicals overflowing - Eye injuries from splashes - Slips from oil spillage - Strains and sprains from lifting and moving Containers of oil. |  |  |
| Procedure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Steps |  | Instructions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 Plan the work |  |  |  | - ONLY authorised and trained are allowed to use the Open Fryer - Ensure the area you will work in is clear of hazards. - Ensure you have the PPE (gloves, apron, closed shoes) required for the job and you know how to wear it properly. - If a shared pair of oven gloves is used. Team members should always wear clean, dry inner gloves when using. - Check all adjustments and settings before commencing any cooking operations - Ensure the stock you will handle is prepared for cooking and easy to move (on trays etc). - Always keep your equipment as clean as possible. Cleaning grease off the deep fryer will not only help prevent fires, but it is also a tick in the hygiene box too. - Always have a wet chemical or fire blanket in close proximity to your commercial deep fryer in case of emergencies. |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 Conduct the work |  |  |  | - Wear your PPE when working with hot substances - Always use a thermometer to check the temperature of the oil and never check the heat using your hand. - Set the thermostat for most foods between 170-180 degrees Celsius. Melt oil slowly at a low temperature. - Use a frying basket or tongs to lower and remove food from the oil. Always lower food into the fryer and never throw it in, as splashing oil that touches skin is dangerous and painful. When removing a basket from the fryer always use protective gloves. Never use copper or brass utensils - Always shake water and ice from food before frying and make sure condensation does not drop into oil pot - NEVER leave the food when it is frying - Keep your workplace clean and grease-free will prevent slipping and unnecessary accidents. - Turn off fryer when not in use. - BEWARE of latent heat. The fryer will hold heat for quite some time. |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 Manual oil filtering |  |  |  | - Follow the manufacturer’s instructions for the oil filtering system making sure to bring any equipment you need to the fryer before you start. - If the oil is too cold to drain easily, reheat it briefly and agitate with the fryer basket for no more than one minute. - Switch the appliance off and check the temperature again before emptying. - Do not dispose of waste oil down the drain – The store has a disposal process in place that complies with environmental legislation. - Clean up any spillages immediately. - Make sure floor areas around equipment are completely clean and dry to avoid slip risks (see also Preventing slips and trips in kitchens and food service). |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 Cleaning procedure |  |  |  | - Turn off the appliance, and the power supply at the wall socket for electric appliances and the on/ off control for gas appliances. - Wear suitable protective equipment, including eye protection (if appropriate). - Check that other activities will not be put at risk by the cleaning activity. - Check that the oil has been thoroughly drained and that there are no spillages that may cause slipping. - Remove loose debris from the internal surfaces. - Thoroughly wash all internal and external surfaces with applicable cleaning chemicals and check for any leaks. - For stubborn residues, fill the fryer with your recommended cleaning agents and leave or simmer according to instructions. - Do not leave the fryer unattended or allow it to boil as this may cause it to cascade liquid onto the floor, causing additional scalding and slipping hazards. - Drain the appliance and rinse thoroughly with plenty of water. - Dry all internal surfaces and make sure there is no water left in the fryer. - Check the drain valve is closed and working properly, then refill and switch on as required. - When refilling the fryer with oil, the oil container must be lifted with 2 people and where possible, use smaller containers. - Do not overfill the fryer. Follow the manufacturer’s guidelines. - Clean up any spillages immediately. - Make sure floor areas around the equipment are completely clean and dry to avoid slip risks. |  |  |  |  |  |  |  |  |  |  |  |  |

| Training | trainee I acknowledge and understand the SWP and the training I have received. Trainer: The trainee has been told and shown what a compliant standard looks like when conducting this task safely, the key risks have been emphasised, and the requirements of the process have been clearly communicated. |
| --- | --- |
| Trainee Name: | Trainee Signature: Payroll Number: Date: |
| Trainer Name: | Trainer Signature: Payroll Number: Date: |
| Trainer Name: | Trainer Signature: Payroll Number: Date: |
| Trainer Name: | Trainer Signature: Payroll Number: Date: |
| Trainer Name: | Trainer Signature: Payroll Number: Date: |
| Trainer Name: | Trainer Signature: Payroll Number: Date: |