| Purpose: |  |  |  |  | To ensure the safe operation and use of the (Add Model/Make) Pallet jack at IFP |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Responsibilities: |  |  |  |  | Authority to operate |  |  |  |  |  |  |  |  |  |  |  |
| Personal Protective Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety Glasses | Gloves |  |  Safety Footwear |  |  | Ear plugs/Muffs | Face shields |  |  High Visibility Vest/Clothing | Safety Hat | Safety Harness | Protective Clothing |  | Safety Mask |  OTHER Cold environment protection as required | NO PPE REQUIRED |
| Pre-Start Checks |  |  |  |  |  |  |  |  |  |  |  |  | Potential Hazards |  |  |  |
| - Completed IFP Competency process * - Dressed appropriately for work with correct PPE and no loose clothing - Australian standard safety boots - Fit for work - First aid kit on hand - Pallet Jack prestart completed: - Inspect the pallet jack in general to ensure attachment points in good condition - Work area ready, no spills and access clear to and from charging area. - Familiarise yourself with the controls before using the pallet jack and ensure they are undamaged and responsive - Observe the wheels to ensure they are not worn out and no cracks are present. * Mandatory requirements. DO NOT operate the pallet jack without these controls in place. |  |  |  |  |  |  |  | Add picture of pallet jack |  |  |  |  | - Manual handling/repetitive strains - Unstable/overloaded loads - Incorrect staking - Objects fall off tynes - Tip over due to sharp turning - Tip forward due to travel with mast high - Failure of plant resulting in loss of content, loss of load - Damaged pallets/bins - Poor visibility/ obstruction/ blind spots - Travelling turning on gradients - Pallet Jack impact on structure - Rain/spills/ causing slippery conditions - Poor work methods/practices - Battery explosion/Chemical burn - Other vehicle traffic - Untrained in safe work procedures - Pedestrians/bystanders |  |  |  |
| Procedure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Steps |  | Instructions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 Plan the work |  |  |  | - Ensure the pre-start checks listed above have been completed. - Ensure the work area is in a suitable condition for the proposed tasks (see hazard list) - Familiarise yourself with the controls before using the pallet jack and ensure they are undamaged and responsive. |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 Operational condition checks |  |  |  | - Ensure the pallet jack pre-start inspection is completed and signed off by the Store Manager. - Undertake your own checks before operation: - Ensuring the pallet jack is on flat ground - Inspecting the pallet jack’s body for damage and hydraulic for breakages and leaks - Observe the wheels to ensure they are not damaged and no cracks present - Checking safety devices. - Verifying that you have 360-degree visibility before operation - Removing loose items from the pallet jack - Finally check steering, and brakes before operation. - Record and report any faults to your Store Manager. - Tag out for prohibited use if any safety features are non-functional. |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 Transporting Loads |  |  |  | - View the area and surrounding before moving off, ensure it is safe to commence work. - While moving around the work area pay attention to team members on foot and ensure you know their positions. Cease operation if persons on foot approaches the machine - Ensure you follow the guidelines of signage for each work area. - If you are carrying a large load that obstructs your view ask a coworker to guide you - Maintain speed limits depending on the environment and reduce speed when in doubt. Especially travelling through doors - Keep the load level at all time - Make sure the load is evenly distributed. - Never exceed the advised capacity. - Move the load slowly to ensure safety in case your surroundings change. - Utilise two people for heavy loads - one person should push, while the other pulls. You should always push the load (never pull the load) - When going down on an incline go in reverse. - Make sure the pallet jack is lowered fully before moving the jack out. - Keep yours and others feet clear when lowering pallet or when reversing the pallet jack. - Be wary of pinch points to avoid injuries to your hands \ - ALWAYS travel in a rearwards direction, whether loaded or empty. - Lower forks and position under load, and lift forks to raise load from ground. - Move slowly; travelling at a steady speed to suit load size and other personnel working in your area. - Sound the horn when approaching walkways, blind corners and keep an eye out for moving personnel. - To brake, lower the tiller to horizontal, releasing the selector/speed control or rotating in the opposite direction of travel. - To stop, lower the forks and reverse out slowly from the load. - Release the operating arm, lower the forks fully to the floor and turn off. - Remember, when you move the tiller in the vertical (or uppermost) position, the parking brake will be automatically activated. - Replace pallet jack into storage area. |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 Shut down and park |  |  |  | - Once work is completed, return the Pallet Jack to the designated parking area. - When parking - Use the handbrake - Lower forks to the ground - Tilt so tips touch - If any faults have been identified while working, record and report them. - Tag out the machine for prohibited use if any faults are found with safety features. |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 Recharge |  |  |  | - Return the Pallet Jack to the recharging station. - Parked position tynes tilted forwards - Follow the charging SWP to correctly and safely recharge the machine. |  |  |  |  |  |  |  |  |  |  |  |  |
| Dos and Don’ts |  |  |  | - DO NOT operate equipment without wearing appropriate PPE. - DO NOT use a jerking action to get a loaded pallet moving. - NEVER place your feet under a pallet jack. NEVER exceed the pallet jack rated load capacity. - NEVER pull a loaded pallet jack, especially - DO NOT use the pallet jack to transport person’s standing/sitting on the pallet, load or an unloaded pallet jack. - DO NOT use faulty equipment. Immediately report any suspect equipment & apply a DO NOT USE tag. |  |  |  |  |  |  |  |  |  |  |  |  |

| Training |  | trainee I acknowledge and understand the SWP and the training I have received. Trainer: The trainee has been told and shown what a compliant standard looks like when conducting this task safely, the key risks have been emphasised, and the requirements of the process have been clearly communicated. |  |  |
| --- | --- | --- | --- | --- |
| Trainee Name: |  | Trainee Signature: Payroll Number: Date: |  |  |
| Trainer Name: |  | Trainer Signature: Payroll Number: Date: |  |  |
| Competency | trainee: I have the knowledge and skills to undertake this task confidently and safely. Trainer: The trainee has demonstrated confidence, understanding and competency in conducting this task safely. |  |  |  |
| Trainee Name: |  |  | Trainee Signature: | Date: |
| Competent Person: |  |  | Competent Person Signature: | Date: |