| Purpose: |  |  |  |  | To ensure team members participate good safety practices whilst using the KP Tenderiser . |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Responsibilities: |  |  |  |  | Butchery Personnel |  |  |  |  |  |  |  |  |  |  |  |
| Personal Protective Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Safety Glasses | Gloves |  |  Safety Footwear |  |  | Ear plugs/Muffs | Face shields |  | High Visibility Vest/Clothing | Safety Hat | Safety Harness | Protective Clothing | Safety Mask |  | OTHER Cold and Weather protection as required | NO PPE REQUIRED |
| Pre Start Checks |  |  |  |  |  |  |  |  |  |  |  |  |  | Potential Hazards |  |  |
| - Dressed appropriately for work with correct PPE - Closed toed non slip shoes - Work area clear to operate safely - Floor clean and free of spillages - Fit for work - First aid kit on hand |  |  |  |  |  |  |  |  |  |  |  |  |  | - Contact with serrated tenderising blades - Infeed and outfeed conveyors - Electrical hazards - Manual handing (repetitive tasks) |  |  |
| Procedure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Steps |  | Instructions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 Plan the work |  |  |  | - DO NOT operate the machinery without training - Ensure you are wearing any required PPE (apron, closed shoes), roll up sleeves, all loose clothing and tie back loose hair and clothing. - Check the tenderiser, ensure it is clean, the blades have been correctly installed and operational with no blockages. - Ensure any electrical leads are in good condition and not in a position to be cut or damaged. - Ensure you have enough room for the products and trays or containers ready to receive the meat. - Ensure easy access to stop button |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 Conduct the work |  |  |  | - Feed meat into chute using tongs. Always keep your hands out of the opening and behind the guards. NEVER REACH INTO the Tenderiser or force the meat into the Blades. - Once work is complete turn off Tenderiser - Wash all parts by hand and dry thoroughly. - Protective/cut-resistant gloves must be used whenever handling the Tenderiser Blades. |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 Cleaning |  |  |  | - Power is turned off - Remove blades - Wipe down machine |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 Safety Tips |  |  |  | - KEEP FINGERS CLEAR OF THE BLADES AT ALL TIMES. Failure to do so may result in personal injury. - DO NOT press or push meat into the Chute with your fingers or any other object. Doing so may cause personal injury and/or damage to the unit. - DO NOT immerse the Blades in sudsy water where they cannot be seen - NEVER use fingers to scrape food away from the Tenderizer while in operation. SEVERE INJURY MAY RESULT. - DO NOT push the Tongs or other utensils into the Blades. - NEVER LEAVE THE Tenderizer UNATTENDED. - NEVER use any accessories or parts from other manufacturers. - DO NOT attempt to tenderize or slice meat with bones or other hard items. Damage to the machine will occur. - REMOVE ties, rings, watches, bracelets, or other jewellery before operating. |  |  |  |  |  |  |  |  |  |  |  |  |

| Training |  | trainee I acknowledge and understand the SWP and the training I have received. Trainer: The trainee has been told and shown what a compliant standard looks like when conducting this task safely, the key risks have been emphasised, and the requirements of the process have been clearly communicated. |  |  |
| --- | --- | --- | --- | --- |
| Trainee Name: |  | Trainee Signature: Payroll Number: Date: |  |  |
| Trainer Name: |  | Trainer Signature: Payroll Number: Date: |  |  |
| Competency | trainee: I have the knowledge and skills to undertake this task confidently and safely. Trainer: The trainee has demonstrated confidence, understanding and competency in conducting this task safely. |  |  |  |
| Trainee Name: |  |  | Trainee Signature: | Date: |
| Competent Person: |  |  | Competent Person Signature: | Date: |